H&S plan
Health and Safety

Project information

| Site nummer | : | L |
| --- | --- | --- |
| Location number or x/y coordinates | : |  |
| Order number | : |  |
| Provider | : |  |
| Version number | : | 2 |
| Date | : |  |
| Status | : |  |

Version management of this H&S plan

| version: | Drafted by: | Date: | Initialled: | Changes: |
| --- | --- | --- | --- | --- |
| 0 | Diann van der Laan | 06-07-2020 |  | New Version |
| 1 | Roan van Merode | 09-07-2021 |  | Added risks regarding the use of cherry picker |
| 2 | Mark Smith |  |  | Updated Information |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

| validation and distribution of this H&S plan: (Manual H&S plan § 4) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Party: | Name: | Function: | Assessment: |  |  | Initialled: | Remarks: |
|  |  |  | A | AV | NA |  |  |
| DAEL | R. van Merode | QHSE Manager |  |  |  |  |  |
| DAEL | Mark Smith | Cordinator |  |  |  |  | Adjust plan to the according risk and measures. read before work |
| DAEL |  |  |  |  |  |  |  |

A = accepted
AV = accepted but subject to remarks 
NA = not accepted, resubmit, see remarks

# How this plan relates to other required documents

This part of the document describes which HS&E measures are implemented for this project in accordance with the legal requirements of the Dutch Working Conditions Act (Arbowet) and its attendant regulations, the requirements laid down in the contract and the project-related documents referred to in the contract, and the rules and procedures of the principal and the contractor.

In addition to this document the following documents apply:

- Manual Monet H&S Plan

- The contract between the principal and the contractor

- Monet “Safety Regulations for Working at KPN, T-Mobile and VodafoneZiggo locations”

- Monet leaflet “Working safely at mobile communication locations; Guidelines, risks and precautions”

- The contractor’s own procedures and guidelines

- Any requirements set by the landlord

- The current site safety RI&E

# Project Data

## General Information

* Required fields

| Provider* |  |
| --- | --- |
| Site number* | L |
| Location number of x/y coordinates |  |
| Order number |  |
| Address * |  |
| Town* |  |
| Company name |  |
| Type of work / structure* | New construction Modification Management and maintenance |
| Type location* | Flat roof Antenna pole Silo Windmill |
|  | Church tower Chimney High Voltage Pylon (HVP) |
|  | High Voltage Tower High Voltage Station Indoor |
|  | Billboard Other, specify: |
| Start of work | dd-mm-yy |
| Scheduled end of work | dd-mm-yy |
| Type of work* (describe activities) For TenneT describe activities day by day. |  |

# Introduction Training (project specific risks and toolbox)

| Information: |  |  | Kopie naar: |  |
| --- | --- | --- | --- | --- |
| Site number | : | L |  | Project supervisor |
| Location number | : |  |  | Direct spervisor |
| Date | : |  |  | H&S Official |
| Trainer | : |  |  | Principal (if arranged) |
| All employees must receive a start-of-work introduction using this form before starting work each day. The following individuals present declare that they have understood the instructions given here and will act accordingly. |  |  |  |  |

Attendants Introduction Training:

| Present: |  |  |  |
| --- | --- | --- | --- |
| DATE | NAME | COMPANY | SIGNATURE |

All marked topics must have been discussed!

| 3.1 Discussed topics: |  | Remarks: |
| --- | --- | --- |
|  | Safe access to the location and specific points of interest. | Site specific risks (see Site RIE): Prevent access by third parties / unauthorized persons. Additional H&S rules (see 3.2) |
|  | Availability Safety Regulations for Working at KPN, T-Mobile, Tele2 and Vodafone locations. | Must be available to alle site workers. |
|  | Safety passports / training ok? | All safety training courses and certificates can be found in the AFAS Pocket app. |
|  | Required personal protection equipment. | Work clothes, safety shoes, (climbing) helmet, safety glasses, hearing protection and fall protection. |
|  | Additional personal protection equipment. | Hearing protection, respiratory protection, flame-retardant clothing. |
|  | On-site emergency services (first aid) in conjunction with locally establishes procedures (tune who first aiders are). | In case of accident, follow the emergency instructions in the general health and safety plan. A first aid kit should be kept at or near the workstation. |
|  | Regulations regarding smoking, alcohol, drugs, and medications. | Smoking only on purpose indicated. Completely sober at work. Also watch out for drug use. |
|  | Appointments sanitation (toilet, hygiene). |  |
|  | Order and cleanliness. | Material together within the safe area. Tools never be left unattended, packaging clean up immediately. |
|  | Slips, trips, falls. | See order and cleanliness. Note also added on rain, frost, snow and hail. Beware of algae. Sprinkle if necessary, sand or salt on walkways. |
|  | Shock hazard to low or high constructions. | Wear a helmet of ‘bump cap’ and highlight outstanding or lower parts with black / yellow belt. |
|  | Safe storage and use of hazardous substances. | Use containers to prevent runoff. Storage in accordance with regulations. Refer to Safety Data Sheet. |
|  | Separate collection of waste. | Wood, paper/cardboard, metal, electronics, chemical and household waste. |
|  | Prevent environmental pollution and damage to flora and/or fauna. | Note nuisance by noise and vibration. Perform work as much as possible in consultation with landlord. If necessary, warn residents in advance. |

Site specific HIGH risks

| In most cases, the standard company RI&E will identify any possible risks and safety measures. HOWEVER, any site-specific HIGH risks (with explanatory remarks if necessary) must be recorded here and the specific safety measures added (see Manual H&S Plan, section 5.3) |  |  |  |
| --- | --- | --- | --- |
| 3.2 Specific risks Landlord |  |  | Specific measure(s) / business rules |
|  | Subject work on/near a high voltage installation |  | See further instructions below 3.3. |
|  | Special access procedures |  |  |
|  | Work permits |  |  |
|  | Cellnex towers (specific measures applicable) |  |  |
|  | Work along the road |  |  |
|  | Work along the track/railroad |  | Training required. Access only with access pass ProRail. |
|  | Held Toolbox meeting(s) (specific risks) |  |  |
| Remarks: |  |  |  |

| 3.3 Working on/nearby high voltage installations |  | Specific measure(s) / business rules |
| --- | --- | --- |
| Applicable documents: - Regulations Antenna Sites TenneT (Voorschriften Antenne-opstelpunten TenneT) (GS-REM 16-0866). - Safety regulations of the grid operator - The drawings approved by the owner. |  |  |
|  | Working near high voltage, electrocution hazard. | Work permitted only under supervision in accordance with NEN3840 |
|  | Climbing a high voltage power pole (HVP) or tower | Leverage existing permanent (inspected) lifeline (Latchways). If the Latchways lifeline is not properly inspected, a temporary lifeline is applied (using the climbing technique with double folding hooks). Required specific fall protection equipment: Y-line with folding hooks, Latchways runner, positioning line, climbing helmet. Rescue equipment presence, ready to use, near the pole body. |
|  | Working on a HVP or high voltage substation and/or installation of mast reinforcements. | Start Work meetings and work according to work plan as required by the grid operator. Only by persons in possession of personal certificate “Attainment work on or near powerlines 2.0”/ Line worker. |

|  | Performing telecom operations near power lines. | Start Work meetings and work according to work plan as required by the grid operator. Only by persons in possession of personal certificate “Attainment work on or in the vicinity of powerlines 2.0”/ Telecom specialist. |
| --- | --- | --- |
|  | Working with a cherry-picker on or near a high voltage pylon or power line. | - Supervise railway by a works manager (NEN3840). -There should be included a TRA/essay drawing of these activities. -This work is carried out by a Line worker. |
|  | Working with a crane with workplatform on or near a high voltage pylon or power line. | - Supervise railway by a Works manager (NEN3840). -There should be included a TRA/essay drawing of these activities. -This work is carried out by a Line worker. -The Workplan Crane with workplatform is has been completed and approved and present at the H&S Plan. |
|  | Hoist with a mobile crane on or near a high voltage pylon or power line. | - Supervise railway by a Works manager (NEN3840). -There should be included a lifting plan / H&S sub-plan / essay drawing of the lifting operations. -This work is carried out by a Line worker. |
|  | Hoist with a hoisting winch on or near high voltage pylon or power line. | - Supervise railway by a Works manager (NEN3840). -This work is carried out by a Line worker. |
|  | Hoist manually in the pole body at or near a high voltage pylon or power pole line. | - Supervise railway by a works manager (NEN3840). - This Work is carried out by a Telecom Specialist. - Last hoist with several people to reduce the physical load. |

| 3.4 Working at heights |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Within two meters from the eaves. |  | Apply Hard deposition of at least 4 meters in length or an arrest (short-term work) |
|  | 2. Working in the 2-4-meter zone, the standard safety measures apply. |  | • The 2 meters boundary flag with red / white necklace or ribbon • The 2 meters zone must not be entered without fall protection |
|  | 3. Roof slope in access of > 15° |  | • Appropriate fencing places. If this is not possible, use personal fall arrest • Silos that are not equipped with a safe railing with toe board: always use fall protection! |
|  | 4. Initial installation of fall protection equipment. |  | Fall hazard should be prevented by the following methods in the order of preference: • Places of (temporary) fences (within 2 meters of falls) or marker (between 2 and 4 meters of falls) • Use of lift (minimize use of ladders) • Use tap with treating tank • Personal Fall Protection • Saving organize height |
|  | 5. Installation of cable ducts. |  | • Use cherrypicker |
|  | 6. Installation of earth leads. |  | • Use cherrypicker |
|  | 7. Installation of antenna pole, antennas, feeders etc. |  | • Use of cherry picker for installation work (minimizing use of ladders) |
|  | 8. Risk of falling objects. |  | • Cordon off the drop zone • Secure tools and materials with lanyards |
|  | 9. Working from a (landing) staircase. |  | • Work in accordance with decision tree "Working on a ladder" • Use only approved own ladder • Place the ladder at the right angle and, if present, use ladder bracket • When operating in a mast, secure the ladder to the top and to apply edge protection • Both hands free for climbing the ladder (if necessary material / tool bag) • The ladder is not permitted for standing times of more than 4 hours |
|  | 10. Working with (rolling-) scaffolding. |  | • Construction scaffolding should be erected only by qualified personnel or contractors • Do not access construction scaffolding until released for use (scaffolding certificate) • Follow manufacturer’s instructions when erecting modular (mobile) scaffolding (do not leave out parts) • Take precautions against traffic collisions • Check stability of mobile scaffolding after moving |
|  | 11. Rope access (working form safety lines) |  | • Restrict rope access to qualified personnel (IRATA of equivalent) • Prepare a workplan based on separate task risk analysis for the work to be done and submit work plan to principal for acceptance • Prepare for emergency operations in high locations |
|  | Other: |  |  |
| Remarks: |  |  |  |

| 3.5 Horizontal and vertical transport |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Hoisting with mobile crane (materials) |  | • Use a certified crane operator • Cordon off the drop zone. • Wear hard hats during lifting operations • Measures against collision hazard |
|  | 2. Working from a crane with workplatform |  | Apply Workplan Crane with workplatform • Always use fall protection • Communication secured by direct view or use of walkie-talkies • Wear a helmet in the work tray • Pay attention to the topicality of the inspection of the platform • Register the activities at iSZW via https://www.inspectieszw.nl/ reporting/werkbakken-enplatformen |
|  | 3. Hosting a winch |  | • Use only approved winch • Operation of the winch only by suitably trained staff • Stop lifting zone (DZ) • Wearing helmet during lifting operations |
|  | 4. Hoisting manually |  | • Use only approved lifting equipment (hoist and pulley)) • Secure lifting line and load during the takeover of the load at height • Wear gloves during lifting • Lifting load at the end of the hoist line must not weigh more than 23 kg. (with a single pulley the load must therefore up to 46 kg) • Cordon off the lifting zone (DZ) • Wear helmet during lifting operations |
|  | 5. Working with a cherry picker |  | • Only to be operated by expert employee trained in accordance with VCA requirements. • Pay attention to good stamping (stability of the substrate) • Always take test flight without a person in the box (if possible 360 degrees) • Always apply fall protection • Take measures against the risk of collisions. |
|  | 6. Attaching loads, using eyebolts and other lifting equipment. |  | Work according to instruction "Vertical transportation" (see Annex to this manual) |  |
|  | Other: |  |  |  |
| Remark: |  | Work in accordance with Arbo catalogue “Vertical transport” |  |

| 3.6 Physical factors |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Working near live electrical components. |  | • Work on electrical installations in operation is prohibited unless they are safe to touch. • Switch off the installations and make sure they cannot be accidentally switched on (lock main switch, check safety by measuring voltage) |
|  | 2. Electromagnetic fields (EMV)exposure standard or unknown antennas |  | • If the antenna RI&E or readings give reason to expect excessive EMF levels, the antenna must be switched of or its output power reduced • Do not work in front of an operating antenna • Do not stay in front of an operating antenna for longer than one minute • Maintain a distance of at least five metres from any operating mobile telephony antenna |
|  | 3. Lasers (optical links) > class 1M |  | • Do not work on cables connected to operating transmission equipment. |
|  | 4. Noise (e.g. church tower bells, machinery, drilling) |  | • First source approach: eg turn off church bells/machines! • Use ear defenders |
|  | 5. Inadequate lighting workstation |  | • Apply spotlights |
|  | 6. Injury when using power tools such as grinder drill, mill, etc. |  | • Use only approved machines • Check drilling, grinding, sawing, etc. machinery on good condition (also sufficiently sharp) and secure it tightly with included tools. (note expiration date on grinding wheels) • Clamp work piece and hold tools with both hands. • Wear safety goggles, ear defenders |
|  | 7. Cold / heat stress (weather, ovens) |  | • Wear suitable clothing. • Adapt working hours and rest times. • Take sufficient hot or cold drinks. • Use sun protection, lip balm etc. |
|  | Other: |  |  |
| Remarks: |  |  |  |

| 3.7 Hazardous materials |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Handling and storing hazardous materials |  | • Store hazardous materials in a suitable (original), closed container and make sure the container cannot fall down or over • Do not store materials together if they can interact chemically • Make sure the container is properly labelled (H and P texts and hazard symbols) • Make sure the product information sheets of the materials are kept on site so adequate emergency treatment can be given if necessary • When handling hazardous materials, use suitable personal protection (gloves, goggles, face mask, respirator) to guard against the risks listed on the container or the product information sheets |
|  | 2. Industrial products (near workstation) |  | • If there is a risk of contact with (or inhalation of) industrial products coming from third-party installations, these installations must be rendered safe by their owner or manager • If this proves impossible, use personal protection suitable for the composition of the hazardous materials involved (obtain information from the owner or manager of the installation) |
|  | 3. Flue gasses (near workstation) |  | • If there is a risk of exposure to flue gases from operating installations, these installations should preferable be switched off • If this proves impossible, use breathing protection suitable for the composition of the flue gases (obtain information from the owner or manager of the installation) |
|  | 4. Riske of exposure to particulate matter (quartz, hardwood) during grinding, drilling, sawing, milling, etc. |  | • Preferably use machines with dust extraction (TNO approved) • Or work by wet drilling, milling or heels in floors and walls (with suitable equipment) • If exhaust ventilation is not possible or effective use respirators P3 |
|  | 5. Risk of exposure to asbestos |  | • If the presence of asbestos is suspected, stop working and notify the supervisor at once. Call in a certified company or qualified asbestos supervisor • Test a sample to ascertain the nature of the material • Have any asbestos removed by a certified asbestos removal company • Do not enter the location until it has been released for use • Record any possible exposure to asbestos in the employee’s file and order a medical examination |
|  | 6. Fibrous materials (glass wool, rock wool, e.g.) |  |  |
|  | 7. Opening and closing cable conduits |  | When opened: • Pay attention to possible asbestos-containing material. When in doubt, do not start, report! • Use a dust mask to prevent inhalation of dust • When removing the mechanical seal, the cables should be disconnected of power • When using manual removal of the seal, do not use chisels or knives that can damage the cables By sealing: • Sealing should only be performed by trained personnel • Only approved / use prescribed sealant |
|  | 8. Risk of exposure to polluted soil |  | See CROW 132 Working with or contaminated soil • In case of suspected contaminated soil stop the work and warn supervisor immediately • Working to CROW400 "Work with or in contaminated soil” • Contaminated soil should be transported form the site by a specialized remediation company |
|  | Other: |  |  |
| Remarks: |  | Remarks: |  |

| 3.8 Biological agents |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Legionella (e.g. form wet cooling tower) |  | • Legionella can occur in pipes and hoses containing stagnant or slowly flowing warm water (up to 65 ℃) • If work needs to be done on such an installation, or if there is any other risk of exposure to the contents of such installations, the installation must be rendered safe • If this proves impossible, breathing protection of at least class P2 must be used |
|  | 2. Animal droppings (direct contact; e.g. rats, pigeons) |  | • If the location is highly polluted, call in a specialist company • Use breathing protection, gloves and overalls • Do not eat or drink on site. Wash or shower after work and wash hands well before eating or drinking |
|  | 3. Insects (ticks, processionary caterpillar, wasps’ nests, e.g.) |  | • Wear full-body clothing (long sleeves end legs) and add hand and head protection if necessary • Call in a specialist to remove a nest (do not attempt to remove a wasps’ nest yourself) • If you get bitten by a tick, use a tick extractor to remove the tick as soon as possible. Do not attempt to stun a tick with alcohol or anything else • If a red ring appears around the tick bite at a later date, you may have contracted Lyme’s disease, so seek medical help immediately |
|  | 4. Irritant plants (skin contact; e.g. giant hogweed) |  | • Avoid skin contact with any plant identified as irritant • If necessary, cut back plants before starting work, wearing protective clothing, gloves, eye and breathing protection |
|  | 5. Biological matters (near workstation) |  | • Prevent viral or bacterial infection by isolating the sourc • If this is impossible, use personal protection (seek expert information regarding the required means of protection) |
|  | Other: Installing batteries |  | Use specified insulated tools mentioned in Reconstruction guide BoostLi Energy Storage Module (Netherlands, TMNL) |
| Remarks: |  |  |  |

| 3.9 Physical factors |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Climbing (e.g. double hooks) |  | • Climb at a steady pace and remain concentrated • Plan the work in detail in advance (e.g. required tools) to avoid unnecessary climbing • Avoid long stretches of work in an uncomfortable position (take breaks) |
|  | 2. Manually moving heavy objects |  | • Use lifting aids whenever possible • Do not lift more than 23 kg on your own • If necessary, ask one or more colleagues for assistance (coordinate activities) |
|  | Other: |  |  |
| Remarks: |  |  |  |

| 3.10 Environmental hazards |  |  | Specific measure(s) |
| --- | --- | --- | --- |
|  | 1. Excavation work |  | See CROW 500 “Damage to cables and pipes” • Working to CROW500 "Damage to cables and pipes" • Caution danger of collapse • Stop the excavation to prevent the risk of falling • Beware of possible contaminated soil (if in doubt supervisor immediately warn) • Excavators only be operated by staff with an operating certificate |
|  | 2. Working in confined space |  | • Always have an oxygen level test done and if necessary add an oxygen supply or use active respiration • Depending on the nature of the enclosed space, additional safety measures must be taken as listed in the work permi • No work may be carried out without a work permit • A manhole watch must be present at all times • Any emergency and evacuation procedures must be prepared for with equipment and trained personnel kept available |
|  | 3. Little movement at the workstation |  | • Take sufficient take breaks if there is a need to work longer in the same position • If necessary, use positioning means to support posture • Wear a helmet or bump cap, knee pads, etc. at risk of impact |
|  | 4. Working in the engine room of a lift |  | • In case of rotating parts the elevator should be switched off or the rotating parts should be shielded according to the Machinery Directive • Do not place any tools or materials on the machine / shield • Take precautions against fire (fire extinguisher, in grinding, welding, etc.), in this space • Ensure good evacuation possibilities |
|  | 5. Risk of fire, explosion (ATEX environment) |  | • Open fire, grinding or welding operations are not permitted in an environment with an increased risk of fire or explosion • If there is an explosion risk (ATEX environment), any work must be carried out in accordance with H&S Directive, section 3.1.2a, explosive atmospheres (Arbobesluit, 3.1.2a, explosieve atmosferen) and the ATEX directive |
|  | 6. Work near or on the water (rescue) |  | • Prevent risk of falling by using temporary or permanent railings or safety harnesses • Have rescue gear available on site |
|  | Other: |  |  |
| Remarks: |  |  |  |

# Contact Information

| Information | Principal | Contractor | Landlord / company ** |
| --- | --- | --- | --- |
| Company name | ODIDO * | DAEL Telecom * |  |
| Visiting address | Waldorpstraat 60, 2521CC Den Haag * | Honderdland 1001 * |  |
| Postal address | Postbus 16272 * | Honderdland 1001 * |  |
| Postcode | 2500 BG Den Haag * | 2676 Lv * |  |
| Town | Den Haag * | Maasdijk * |  |
| Phone general | +31 614 095 000 * | 0174 523921 * |  |
| Contact | Najim Elboustati * | Jaap Bruin * | * |
| Phone contact | +31 624095959 * | +31 6 20690635 * | * |
| E-mail contact | Abdenajim.elboustati @t-mobile.nl * | Jaap.Bruin@dael.com * |  |
| Supervisor | Walter Palm * | J. Bruin * |  |
| Phone Supervisor | +31 615682030 * | +31 (0)6 2069 0635 * |  |
| Direct Supervisor | Rene Christ | Team leader * |  |
| Phone Direct supervisor | +316 24096813 | * |  |
| H&S Official | Frans Munts * | R. van Merode * |  |
| Phone H&S Official | +31628023823 * | +31 (0)6 2381 0122 * |  |

| Information | Onderaannemer 1 | Onderaannemer 2 | Onderaannemer 3 |
| --- | --- | --- | --- |
| Company name | * | * | * |
| Visiting address | * | * | * |
| Postal address | * | * | * |
| Postcode | * | * | * |
| Town | * | * | * |
| Phone general | * | * |  |
| Contact | * | * | * |
| Phone contact | * | * | * |
| E-mail contact | * | * |  |
| Supervisor | * | * |  |
| Phone Supervisor | * | * |  |
| Direct supervisor | * | * |  |
| Phone Direct supervisor |  |  |  |
| H&S Official | * | * |  |
| Phone H&S Official |  |  |  |

*	Required fields

** 	Specific H&S rules and risks obtained from the above contact person.

| Information | Onderaannemer 4 | Onderaannemer 5 | Nevenaannemer *** |
| --- | --- | --- | --- |
| Company name | * | * | * |
| Visiting address | * | * | * |
| Postal address | * | * | * |
| Postcode | * | * | * |
| Town | * | * | * |
| Phone general | * | * |  |
| Contact | * | * | * |
| Phone contact | * | * | * |
| E-mail contact | * | * |  |
| Supervisor | * | * |  |
| Phone Supervisor | * | * |  |
| Direct supervisor | * | * |  |
| Phone Direct supervisor |  |  |  |
| H&S Official | * | * |  |
| Phone H&S Official |  |  |  |

*	Required fields

** 	Specific H&S rules and risks obtained from the above contact person.

***	Secondary contractors are companies not under the main contractor’s supervision that me be present at the project location to carry out work of their own and which therefore may introduce risks

| H&S Coordinators Main contractor | Name: | Phone: | E-mail: |
| --- | --- | --- | --- |
| V&G Coördinator ontwerp | Mark Smith* | 0638061468 * | mark.smith@dael.com * |
| Vervanger (optie) | Dennis de Vries | +31 (9)6 829 56 814 | dennis.devries@dael.com |
| V&G Coördinator uitvoering | Team Leader * | * | tmnlprocess@dael.com * |
| Vervanger (optie) |  |  |  |
| Werkverantwoordelijk (WV) | * | * | * |

## Project organisation

|  | Landlord / company |  |  | Project Supervisor |  |  |  | Secondary Contractor(s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | site location |  |  | Principal |  |  |  |  |  |  |  |
|  |  |  |  | Walter Palm |  |  |  |  |  |  |  |
|  | H&S Official |  |  | Project Supervisor |  |  |  | Subcontractor(s) |  | Sub- |  |
|  |  |  |  | Contractor |  |  |  |  |  | subcontractors |  |
|  | R. van Merode |  |  | J. Bruin |  |  |  |  |  |  |  |
|  | Construction |  |  |  |  |  |  |  |  |  |  |
|  | H&S Coordinator |  |  | Supervisor op location |  |  |  |  |  |  |  |
|  | Team Leader |  |  | Team leader |  |  |  |  |  |  |  |
|  |  |  |  | Mechanics |  |  |  |  |  |  |  |

# On-site emergencies (see H&S Plan Procedure)

| Alarm numbers: |  |  |  | What to report: - Your name - Location - What happened - Number of victim - Nature of injuries - Is first aid available? 7. Phone number |
| --- | --- | --- | --- | --- |
| 1. On-site emergency services arm number (BHV) of |  |  |  |  |
| 2. General emergency number | : | 112 |  |  |
| 3. Police (not urgent) | : | 0900-8844 |  |  |
| Address Project Location | : |  |  |  |
| Location number/x-y coordinates | : |  |  |  |

| Other information: |  | Name: | Phone: | Address: |
| --- | --- | --- | --- | --- |
| Local Hospital | : |  | (+31) |  |
| Local Doctor | : |  | (+31) |  |

| What to do: REMEMBER: SAFETY FIRST! |  |  |
| --- | --- | --- |
| Accident | - Move any injured persons to a safe place if necessary - Call first aid (on-site emergency services or first aid volunteer). - If necessary call for expert help using the local emergency number; or if there is none, dial 112. - Keep spectators away. - Hand over to the emergency services when they arrive. | A first aid kit must be kept at the workstation. |
| Brand | - Call the local emergency number. - Alert the surroundings. - Close doors and windows - Attempt to put out a starting fire - Hand over to the emergency services when they arrive. | A fire extinguisher must be kept at the workstation is there is a fire risk. |
| Evacuation | - Halt work immediately, but safely. - Leave everything behind. - Follow the shortest escape route as indicated. - Report at your assembly point | Before starting work, ask about the evacuation signal, escape route and assembly point. |

| Persons to be notified as soon as possible: |  |  |  |
| --- | --- | --- | --- |
| Contractor’s contacts |  | Name: | Phone number: |
| Project Supervisor | : | J. Bruin | +31 (0)6 2069 0635 |
| Direct Supervisor | : | Team leader |  |
| H&S Official | : | R. van Merode | +31 (0)6 2381 0122 |
| General Office number | : | Reception | 0174 523921 |
| Work Manager (WM) (OG of ON) | : |  |  |
| Principals Contacts: |  | Name: | Phone number: |
| Project Supervisor | : | Walter Palm | +31 615682030 |
| H&S Official | : | Frans Munts | +31628023823 |